SMALL C COMPILER

Introduction

This package  contains an implementation of  the Small-C compiler
         originally published in Dr.  Dobb's Journal. This compiler, which
         was originally written  to run on Z80 and 8080  based systems has
         been  heavily  modified to  run  on  the  QL. Included  with  the
         compiler  are  two  other  programs specially  written  for  this
         implementation, a Code Generator/Linker  and a Library Generator,
         as well as some library programs. These notes are not intended to
         teach you  how to  program in C,  but to explain  how to  use the
         compiler suite.

The compiler is an integer only compiler handling a subset of the
         complete C  language, a  library is  provided to  handle floating
         point arithmetic and QL input/output.

Any  comments on  these programs,  suggestions for  improvements,
         complaints,  etc  will  be  welcome, but  do  include  a  stamped
         addressed envelope  (or international reply coupons  from outside
         the UK) or you  are unlikely to receive a reply.  The address for
         correspondence is:

15 Edgehill

Llanfrechfa

Cwmbran, Gwent

Tel. 06333 2175

Small-C Compiler

This was written by J.E.Hendrix  and placed in the public domain.
         A description  and source  code of the  compiler (and  many other
         useful programs are  contained in a book "Dr.  Dobb's Toolbook of
         C" published in 1986 by Brady (Prentice Hall). The compiler takes
         C  source  code  and  produces an  intermediate  code  file.  The
         original  compiler produced  output suitable  for a  conventional
         assembler  and linker.  This  implementation  produces a  numeric
         intermediate   code   file   which    is   used   by   the   Code
         Generator/linker.  This approach  was  taken because  it is  much
         faster eg with the original system  it took the compiler about 12
         minutes  to compile  itself,  and  a further  13  minutes for  an
         assembler  to  generate  an  executable program.  With  the  code
         generator  approach the  compiler takes  6 minutes  and the  code
         generator 50 seconds. Large programs  can be compiled in separate
         modules and the object modules linked together into an executable
         program by the code generator  linker. This makes recompilation a
         much quicker process when programs are modified, since only those
         modules which have been changed need to be recompiled.

page 1
         Code Generator/Linker

This has been specially written for  the QL using Small-C itself.
         It  takes  object code  modules  generated  by the  compiler  and
         library  modules produced  by  the  library generator,  generates
         68008  machine  code  for  each module,  and  links  all  modules
         together to produce an executable program which may EXEC'd in the
         usual  way. Executable  programs  are limited  to 64K  (including
         runtime  dataspace)  in size,  if  there  is enough  interest  or
         demand, a version may be  produced which has no size limitations.
         The code  generator tells  you if the  executable program  is too
         large.

Library Generator

Another program written  in  Small-C for  the  QL,  this takes  a
         series of compiled object modules, generated by the compiler, and
         generates a single  library module which may be used  by the code
         generator. This  is typically used for  C functions which  may be
         used  by  several application  programs.  Combining  them into  a
         single library module  is useful because the  code generator only
         selects those  library functions which  are actually used  by the
         object modules, ie irrelevant,  unused functions are not included
         in the  final executable code  file. The standard  library module
         supplied in this package was generated by this library generator.

Files included in this package

cc            C Compiler

cg            Code Generator/Linker

lg            Library Generator

mc_obj        Machine code module                    
                   std_lib       Standard library

stdio_h       Standard definitions

sieve_c       Example program

readme1_doc   See readme1_doc

readme2_doc   Standard library description

default_bas   Change the default device

sort_c        Example program

Running the programs

This is achieved  by using EXEC or a similar  command provided by
         the various toolkits available. Each  program will then request a
         command line.

Command lines

The approach taken with all three  programs is for the program to
         request the input of a command  line, to tell the program what to
                                       page 2
         do. The  command line  will typically contain  a mixture  of file
         names and command line switches  separated by spaces. A file name
         is specified either  by using the full QDOS name  preceded by the
         device name eg  mdv1_xyz_c or by relying on a  default device and
         extension provided  by the program eg  xyz , or a  mixture of the
         two eg mdv1_xyz or xyz_c . A command line switch is recognised by
         the first character being a -  . Details of the command lines for
         the three programs  are given below. Programs written  by you may
         optionally use the command line  facility by using a command line
         switch in  the Code Generator/Linker.  File names on  the command
         line must not be bracketed by quote characters.

A further command line facility not used by the compiler suite,
         but used in the example program sort, is input/output
         redirection, for which see a later explanation.

Compiler command line

This  contains one  or more  C  source file  names with  optional
         command line switches.  The source file must be stored  in a file
         on one of the  standard QL devices eg mdv1_ flp1_  ram1_ etc. and
         the file  name must end  with _c or _C  eg mdv1_xyz_c .  See, for
         example,  the supplied  program mdv1_sieve_c  . Possible  command
         line switches are:

-p        Pause if an error occurs, it then waits until
                             ENTER is pressed, if ESC is pressed the 
                             program aborts.

-m        Monitor progress, this prints the first line 
                             of every C function as it is compiled.

-d/dev    Sets the default device for C source files, 
                             used to overcome the default in the Program 
                             which, as supplied is mdv1_ (this may be 
                             altered, see below)

The first item (or argument) should be a file name, this is taken
         both as  an input and an  output file name. The  output file name
         replaces the _c extension with  an _obj extension. Other switches
         and file  names may occur in  any order. If other  file names are
         included they are compiled into the same object module.

For example, to compile sieve_c any of the following may be typed
         in as the command line:

mdv1_sieve_c -p -m

sieve -m -d/ram1_ -p

sieve

All of these will produce an object module called sieve_obj .

A hyperthetical example such as:

page 3
                   fred -p mary_c flp1_joe -d/ram1_

will attempt to compile  ram1_fred_c , ram1_mary_c and flp1_joe_c
         pausing after  every error. A single  object module ram1_fred_obj
         will be produced, for input to the code generator.

Code Generator/Linker command line

This contains  a mixture  of an output  file name,  object module
         names, library module names and  command line switches. The first
         name on the line is taken as the name of the executable task, the
         others may be in any order,  except that, if you have two library
         modules the  first one may call  functions in the second  but not
         vice versa, similarly for more  than two library modules. If this
         causes problems, combine library modules.

Object module  names must  have the _obj  extension which  may be
         omitted on  the command  line. Library modules  must have  a _lib
         extension which  must be included in  the command line  (the code
         generator  uses this  to distinguish  between object  and library
         modules).

Note that the code  generator expects to see two  of the supplied
         files on the default device, mc_obj  and std_lib . These must not
         be   specified  on   the   command  line,   the  code   generator
         automatically searches  for these and  will generate an  error if
         not available on  the default device. This is done  to reduce the
         amount of typing on the command line.

Possible command line switches are:

-p        Pause on an error, waits for ENTER to be 
                             pressed

-m        Monitor progress, outputs original source 
                             file names

-d/dev    Set the default device

-nc       No command line request to be included in the
                             executable program. Omitting this will result
                             in a command being requested

-c/name   reads a file for the command line, eg   
                             -c/ram1_fred will read file ram1_fred_cmd for
                             a command line, the _cmd extension in the 
                             command file name is mandatory. This    
                             switches, but does override any following 
                             ones and all filenames.

-sn       where n is a decimal number. Sets the   
                             dataspace of the executable program to n. eg 
                             -s2000 sets the programs runtime dataspace to
                                       page 4
                             2000 bytes. Set to 7000 by default.

eg to generate an executable sieve program called sieve, any of  
         the following command lines will do:

mdv1_sieve mdv1_sieve_obj

flp1_sieve sieve -p -m -s1000 -nc                    
                   sieve sieve

Library Generator command line

This contains only object module names, there are no command line
         switches,  library module  names should  not be  included. Object
         module names must have an _obj  extension which may be omitted on
         the command line.  The first name is taken as  the output library
         module name  with the _obj extension  being replaced with  a _lib
         extension. For  example the  supplied library module  std_lib was
         produced with this command line:

std fp qdos

with  files std_obj  fp_obj  and qdos_obj  being  on the  default
         device. Even  if only one file  is being made into  a library the
         Library Generator  must be  used because  it converts  the object
         code into a special format.

Default devices

This  is set to mdv1_ for  all three programs, this may be changed
         by:

(a)  using  the supplied default_bas which is exceedingly slow but
         does  not rely on you having a toolkit, you can probably modify it
         if  you  have  a suitable toolkit both to speed it up and lose the
         RESPR.

(b)  use  an  editor  such  as  Digital  Precision's  The  Editor,
         (use unformatted input mode with overstrike).

the string mdv1_ only occurs once in each program.

Compiler features

The  compiler  is  a  16 bit integer compiler handling a subset of
         the  C language. It will compile code containing the following:

Data  types    char,  int,  extern, 1 dimensional arrays, pointers
                     global and local variables                      
                     variables and arrays may be initialised, default to 0

Only one dimensional arrays can be used.

Statements  if ... else

while

do ... while

page 5
                     for

switch ... case ... default

goto label

return

break

continue

Operators   + - * / % ~ !  <<  >>

<  >  <=  >=  ==  !=

&  ^  |  &&  ||

++  --

=  +=  -=  *=  /=  %=

<<=  >>=  &=  ^=  |=

&  (address of)

*  (indirection)

Only  the  first 8 characters of  a variable or function  name are
         significant, upper and lower case characters are different.

The   final  executable program  must  contain  a function  called
         main()   or main(argc,  argv)  which will  be  the first  function
         executed.   If a  command  line is  requested  by the  application
         program,  the  arguments argc and argv  will be passed  to main().
         Argc  and  argv have the usual  meanings ie argc is  the number of
         arguments  in the command line, argv points to an array containing
         the argument strings.

If  your  program   contains a function called _banner,  this will
         be  executed  before  a  command  line    is requested. The use of
         function  and  variable  names    beginning  with  _  should    be
         otherwise avoided since the system uses several such names.

If  your program contains a function called _console, this will be
         executed  before  any  console window is opened by the entry code,
         this  is  useful  to open your own console window and override the
         default  windows.  Your  _console  function  must  return the file
         descriptor of your console window.

Preprocessor

A simple preprocessor is included which recognises the commands:

#define   Define   a   processor   macro   (see   stdio_h      for
                   examples  of    use),  will  not  handle      parameters
         #include  Include   text   from   another   source   file,   these
                   may  not  be  nested  ie  include  files  may        not
                   include other files

#ifdef    Conditionally   compile   the   following   text      if
                   the       following       macro       is         defined
         #ifndef   Conditional   compilation   if   name   not      defined
         #else     Alternative       in       conditional       compilation
         #endif    Ends a section of conditional compilation

page 6
         Function argument counts

Few  functions expect a variable number of arguments, only two are
         supplied  in  std_lib, printf()  and fprintf().  If these  are not
         used  then the  compiled code may be made more  concise and faster
         by   defining NOCCARGC  at the  start of  a source  file. eg  with
         #define  NOCCARGC The uppercase is essential. If printf or fprintf
         are  used  then NOCCARGC must  not be  defined. Of course  you can
         restrict  those functions  using printf and fprintf  to a separate
         module.

Error reports

These    are  mostly   self explanatory, the  rather cryptic "must
         be  lvalue"  usually means you have  not declared a  variable. The
         error   marker  appears  just  after  the  actual  error,  missing
         semicolons  are often at the end of the previous line. An error in
         an  expression turns off error detection until the next semicolon,
         so there may be more errors on a line.

The  nature  of Small-C, where undeclared  identifiers are assumed
         to  be function names in another module means that some undeclared
         variables   are not  detected  until the  Code  Generator is  run.
         Variables   in other  modules must  be declared  using the  extern
         declaration.

Error  reports  in the Code Generator  refer to the  source module
         and   line  number  where  the  error  was  detected,  and,  where
         relevant,  give  the offending identifier. If  a required function
         is   missing, for  example,  only  the first  reference  to it  is
         reported, further occurrences are suppressed.

Standard definitions

Some  standard  definitions are in a  file called stdio_h  To make
         programs  portable to other compilers, these definitions should be
         used  instead of the constants  they represent. In particular note
         that,  in this implementation, a NULL address is -32768 (Hex 8000)
         and not 0, address 0 actually exists.

Standard C channels

There  are  3 standard channels when a compiled Small-C program is
         started  up  called stdin, stdout and stderr for input, output and
         error  reports  respectively.  These all use the same QDOS channel
         which  is  a console channel. If in your program you close one you
         will  close  all  three! If you wish to use these channels in your
         program  you should use stdin etc as the file descriptor fd in the
         i/o  function calls described in the standard library. You are, of
         course  free  to open more channels up to a maximum of 16 in total
         (including the three already used).

page 7
         Input/Output Redirection

The  standard  channels  stdin  and stdout may be changed from the
         console  window  by  redirection  in the command line. This is not
         used  in  the  compiler itself but may be used in the example sort
         program.  Redirection  is  achieved  by  preceding  the    desired
         filename by < or > or >> eg.

<mdv2_fred  in a command line will take standard input from a file
                    mdv2_fred. Of course this file must already exist.

>mdv2_joe  will send standard output to mdv2_joe having already

deleted any existing file with that name.

>>mdv2_mary will append standard output to the end of a file

mdv2_mary

Example program sort

As  a  further  example of what can be done with this version of C
         an  example  file  sort  program  is included which will sort text
         files  only.  You  will have to compile this first with (possibly)
         the following series of commands and command lines:

exec mdv1_cc

with command line:  mdv1_sort -m

exec mdv1_cg

with command line:  mdv1_sort mdv1_sort -s20000

which  will create a task file named mdv1_sort with a dataspace of
         20000  bytes,  which  is  used  for  a sorting buffer. If the file
         being  sorted  is  larger  than  this, temporary files are created
         with  the  name  and  on  the  device specified in the source file
         mdv1_sort_c.  This  is  set  to  ram1_sort00.$$$  which you can of
         course change as desired if you do not have a ram disc.

To activate sort do:

exec mdv1_sort

with command line:  <mdv1_sort_c -u

which  takes  input  from  file  mdv1_sort_c,  sorts  the   lines,
         discards  duplicate  lines  and displays the rest on the screen. A
         command line:       <mdv1_sort_c >mdv1_ordered -u

will do the same but save the results in file mdv1_ordered.

Note that tabs in the source file are significant.

Possible command line switches are:

-u        Unique, ie discard duplicate lines

page 8
         -d        Sort in descending order

-q        Use quicksort instead of the default Shell sort

-c#       Sort from column # eg -c10 sorts from column 10

-f#?      Sort  from field # where character ? delimits the fields
                   eg -f2* means sort on the second field where asterisks

are used to delimit fields. If no delimiter is    
                   specified white space is used as the field delimiter.

White space is space or tab characters.

_____________________________________________________________________________
File: E:\QL Software\Sprachen\SmallC\readme1_doc
Translated by Quill-View 0.5 Beta (compiled Oct 13 2008)
Copyright 2008 Mikael Strom